# Project Decision Log

Project: Least-Squares Based Aerodynamic Pressure Curve Matching

Decision #001

Title:

Use only upper-surface Cp distributions.

Decision:

Only the upper-surface (solid-line) Cp distributions will be digitized from the NACA source figures.

The lower-surface (dashed-line) distributions will not be used in this project.

Reason:

The project compares one Cp curve with another using SSE, RMSE and maximum |ΔCp|. Using only one surface keeps the comparison consistent, avoids mixing different physical distributions, and simplifies the explainable matching process.

Decision #002

Title:

Correct the shifted x-axis from Gregory & O’Reilly Figure 4.

Decision:

The plotted x-axis in Figure 4 is x/c + 0.05α. Therefore, digitized x-values from WebPlotDigitizer are treated as x_plot, not directly as x/c.

The real x/c value is recovered using:

x/c = x_plot - 0.05 * alpha

Reason:

The source figure shifts each angle-of-attack curve horizontally to make the pressure distributions easier to read. Without this correction, the digitized curves would have incorrect x/c positions.

Decision #003

Use only the common overlapping x/c range of all digitized Cp curves.

Reason:

Some source curves do not visibly extend to x/c = 1.0 in the published figure. No Cp values will be invented or extrapolated beyond the digitized data.

Decision #004

Title:

Treat circle (○) and cross (×) symbols as the same experimental Cp data points.

Decision:

During digitization of Gregory & O’Reilly Figure 4, both circle (○) and cross (×) markers are treated as experimental data points belonging to the same Cp distribution.

The marker shape is ignored during digitization. Only the corresponding (x/c, Cp) coordinates are extracted and stored.

Reason:

The figure legend identifies both symbols as experimental points associated with the pressure distribution. The different marker shapes are used only for graphical clarity in the printed figure and do not represent different datasets, different measurement methods, or different airfoil surfaces.

Ignoring the marker type keeps every digitized Cp curve consistent and ensures that only the physical pressure distribution is used for least-squares curve matching.

Decision #005

Title:

Separate source curves, reference curves and training curves.

Decision:

- The Gregory & O'Reilly digitized Cp curves are treated as Source Curves.

- The Source Curves are used only to generate the 50 Reference Curves.

- The Training Curves will be obtained independently from another reliable aerodynamic source.

- Training Curves will not be digitized from the Gregory & O'Reilly reference figure.

Reason:

This keeps the evaluation independent. The matching algorithm compares unseen training data against a reference library instead of matching curves that originate from the same published figure.

Decision #006

Title:

Use Ladson et al. pressure data as the training-curve source.

Decision:

The training Cp curves will be taken from the Ladson et al. pressure data file downloaded from the NASA/TMR NACA 0012 validation case.

The Gregory & O’Reilly digitized curves will remain the source for generating the 50 reference curves.

Reason:

This keeps the reference data and training data separated. The matching algorithm will compare Ladson training Cp curves against a Gregory-based reference library, instead of matching data produced from the same source.

This also prevents confusion in the code: Gregory data is used to create reference_curves.csv, while Ladson data is used to create training_curves.csv.

Decision #007

Title:

Use a common x/c grid for reference and training curves.

Decision:

Both reference_curves.csv and training_curves.csv use the same x/c grid from 0.01 to 0.94.

Decision:

The matching algorithm compares Cp values only at shared x/c positions. No extrapolated Cp values are used.

Reason:

SSE, RMSE and max |ΔCp| require training and reference curves to be evaluated at the same x/c points. Using a common overlapping x/c range keeps the comparison fair and avoids inventing missing Cp data.